# Setting up your Network

# on Chassis B1, B2, & C1

## Discovering your network

- Open Teams and enter into the Neumont BSIS

- In General go into the IS Blade and VLAN Assignments Excel sheet

- Click on the quarter spreadsheet you are in for example Summer 2022

- On the spreadsheet find your name

  - You will have multiple blades and they may not be next to each other so check the whole sheet.

- Find the ID number of all of your blades

- Your internal network will be 10.0.[lowest blade ID number].0/24

  - For example Chassis D2 Blade 10 will have a network of 10.0.210.0/24

- Your VLAN ID will also be the lowest blade ID Number

  - For example Chassis D2 Blade 10 will have a VLAN of 210

## Setting up your VLAN on the Master Core Switch

- Connect to the Ninja009 wireless network. You can locate the password in the Passwords Excel sheet

- Putty into the Master Core Switch. You can find its IP in the Rack IPs Excel sheet

- Go to the Passwords Excel sheet and locate the username and password Rack B-D Switches & Fabrics

- Login into the Master Core Switch using the credentials you just found

- Enter the following commands

> configure terminal > vlan [your VLAN number] > name [first initial] [last name]-[course code] 0/ - i.e. cburk-bit351 > interface vlan [your VLAN number]-[course code] > ip address 10.0.[your lowest blade id number].1 255.255.255.0 > end > show vlan ¦ include vlan [your VLAN number]

- Verify Configuration

  - Ping the IP address you just set from your computer

#### If verification didn’t work

- Go into teams and open the Adding VLANs to VTP database doc and follow the steps

- Ping the IP address you just set from your computer

## Top of Rack Switches

No configuration is needed. These switches will automatically receive the configuration changes that you made on Master Core switch via VTP.

## Setting up your VLAN on the Chassis Switch

- Go to the Rack IPs Excel sheet in teams

- Locate the IP of the A1 Switch on the Chassis your blade is located on

- Use PowerShell or Putty to connect using SSH to the IP address

- Login using the ISStudent credentials found on the Passwords Excel sheet

- Enter the following commands

  - Example: blades 4 and 5

> configure terminal

> interface vlan [VLAN number]

> name [first initial][last name]-[course code]

i.e. cburk-bit351

> description [first initial][last name]-[course code]

> tagged TenGigabitEthernet 0/[blade slot numbers],41

i.e. tagged TenGigabitEthernet 0/4,5,20,21,41

> end

> show vlan

- Verify that your VLAN has been created

- Locate the IP of the A2 Switch on the Chassis your blade is located on

- Repeat steps 3-5 on A2 Switch

- Verify that your VLAN has been created